# 4-Wheel Robot with Arduino Mega

Session Documentation

Date: February 2026

# Project Overview

This document covers the complete setup and testing of a 4-wheel robot using PG36555126000-19.2KE7 planetary gear DC motors with encoders, controlled by Arduino Mega 2560 and two L298N motor drivers, with HC-05 Bluetooth control capability.

# Components Used

Hardware:

• Arduino Mega 2560

• 4x PG36555126000-19.2KE7 Planetary Gear DC Motors (12V) with Encoders

• 2x L298N Motor Driver Modules

• HC-05 Bluetooth Module

• 12V Power Supply (minimum 5A recommended)

# Pin Connections

## L298N #1 (Motor 1 & Motor 2)

| L298N #1 Pin | Arduino Mega Pin |
| --- | --- |
| ENA | Pin 2 (PWM) |
| IN1 | Pin 22 |
| IN2 | Pin 23 |
| IN3 | Pin 24 |
| IN4 | Pin 25 |
| ENB | Pin 3 (PWM) |
| OUT1/OUT2 | Motor 1 |
| OUT3/OUT4 | Motor 2 |

## L298N #2 (Motor 3 & Motor 4)

| L298N #2 Pin | Arduino Mega Pin |
| --- | --- |
| ENA | Pin 4 (PWM) |
| IN1 | Pin 26 |
| IN2 | Pin 27 |
| IN3 | Pin 28 |
| IN4 | Pin 29 |
| ENB | Pin 5 (PWM) |
| OUT1/OUT2 | Motor 3 |
| OUT3/OUT4 | Motor 4 |

## Encoder Connections

| Encoder | VCC | ChA | ChB |
| --- | --- | --- | --- |
| Motor 1 | 5V | Pin 18 | Pin 30 |
| Motor 2 | 5V | Pin 19 | Pin 31 |
| Motor 3 | 5V | Pin 20 | Pin 32 |
| Motor 4 | 5V | Pin 21 | Pin 33 |

## HC-05 Bluetooth Module

| HC-05 Pin | Arduino Mega Pin |
| --- | --- |
| VCC | 5V |
| GND | GND |
| TXD | Pin 15 (RX3) |
| RXD | Pin 14 (TX3) |

# Power Supply Connections

All components share the same 12V power supply:

• 12V (+) connects to: L298N #1 (12V), L298N #2 (12V), Arduino Mega VIN

• 12V (-) connects to: L298N #1 GND, L298N #2 GND, Arduino Mega GND

Important Notes:

• Remove ENA and ENB jumper caps on BOTH L298N boards

• All GNDs must be connected together (common ground)

• Use breadboard to share 5V for all encoder VCC connections

# Issues Encountered & Solutions

### Issue 1: Motor 3 not spinning when all 4 motors start together

Cause: Power supply current limitation - starting all motors simultaneously draws too much current.

Solution: Start motors sequentially with 50-200ms delays between each motor start.

### Issue 2: Serial Monitor showing garbage characters

Cause: Baud rate mismatch between code (115200) and Serial Monitor (9600).

Solution: Set Serial Monitor baud rate to 115200.

### Issue 3: Motors not responding to Arduino commands

Cause: Missing common ground connection between Arduino and L298N/power supply.

Solution: Ensure all GNDs are connected together.

# Bluetooth Control Commands

| Command | Action |
| --- | --- |
| F | Forward |
| B | Backward |
| L | Turn Left |
| R | Turn Right |
| S | Stop |
| 1 / 2 / 3 | Speed: Slow / Medium / Fast |

# Final Notes

• HC-05 default pairing PIN: 1234 or 0000

• Use Bluetooth RC Controller app from Play Store

• For standalone operation, power Arduino via VIN pin from 12V supply

• Recommended power supply: 12V, 5A minimum for reliable 4-motor operation